Da-Lite®

3100 N. Detroit St.

Warsaw, IN 46582

Phone: (800) 622-3737

Phone: (574) 267-8101

Fax: (877) 325-4832

Fax: (574) 267-7804

E-mail: info@da-lite.com

www.da-lite.com

This MANU-SPEC® utilizes the Construction Specifications Institute (CSI) Project Resource Manual (PRM), including MasterFormat™, SectionFormat™ and PageFormat™. A MANU-SPEC is a manufacturer-specific proprietary product specification using the proprietary method of specifying applicable to project specifications and master guide specifications. Optional text is indicated by brackets [ ]; delete optional text in final copy of specification. Specifier Notes precede specification text; delete notes in final copy of specification. Trade/brand names with appropriate product model numbers, styles and types are used in Specifier Notes and in the specification text Article titled “Acceptable Material.” Metric conversion, where used, is soft metric conversion. This MANU-SPEC specifies manually operated projection screens as manufactured by Da-Lite® Screen Company, Inc.

Revise MANU-SPEC section number and title below to suit project requirements, specification practices and section content. Refer to CSI MasterFormat for other section numbers and titles.

SECTION 11 52 13

MANUALLY OPERATED PROJECTION SCREENS

- GENERAL

  - SUMMARY

    - Section Includes: This Section specifies manually operated projection screens and accessories.

Specifier Note: Revise paragraph below to suit project requirements. Add section numbers and titles per CSI MasterFormat and specifier’s practice.

  - RELATED SECTIONS

    - Section [______].

  - DEFINITIONS

    - Gain: Indication of screen’s luminance or brightness measured perpendicular to screen center and relative to magnesium carbonate block, which serves as standard for 1.0 gain. Higher numbers indicate greater brightness.

    - Viewing Angle: Angle from perpendicular center of screen at which gain or brightness decreases by 50%.

    - Format: Proportion of projection screen viewing area expressed as a ratio of width to height.

      - NTSC or Video Format: 1.33:1.

      - HDTV Format: 1.78:1.

      - 16:10 Wide: 1.60:1.

      - Square: 1:1.

      - [______]

Specifier Note: Article below may be omitted when specifying manufacturer’s proprietary products and recommended installation. Retain Reference Article when specifying products and installation by an industry reference standard. Indicate issuing authority name, acronym, standard designation and title. Establish policy for indicating edition date of standard referenced. Contract Conditions or Section 01 42 19 - Reference Standards may establish the edition date of standards. This Article does not require compliance with the standard, but is merely a listing of references used. Article below should list only those industry standards referenced in this section. Retain only those reference standards to be used within the text of this Section. Add and delete as required for specific project.

  - REFERENCES

    - International Code Council (ICC):

      - International Building Code.

    - Society of Motion Picture and Television Engineers (SMPTE):

      - SMPTE RP 94-2000 Gain Determination of Front Projection Screens.

  - ACTION SUBMITTALS

    - General: Submit listed action submittals in accordance with Contract Conditions and Section [01 33 00 - Submittal Procedures] [______].

    - Product Data: Submit product data, including manufacturer’s technical product data sheet, for specified products.

      - Material Safety Data Sheets (MSDS).

    - Shop Drawings: Indicate dimensions, fabrication and installation details.

    - Samples: Submit [2] [______] [6 inches × 6 inches (152 × 152 mm)] [______] samples of screen finish material.

  - INFORMATION SUBMITTALS

    - Quality Assurance:

      - Test Reports: Certified test reports showing compliance with specified performance characteristics and physical properties.

      - Certificates: Product certificates signed by manufacturer certifying that materials comply with specified performance characteristics, criteria and physical requirements.

      - Manufacturer’s installation instructions.

  - CLOSEOUT SUBMITTALS

    - Operation and Maintenance Data: Submit for products in accordance with Section [01 78 00 - Closeout Submittals] [______].

      - Manufacturer’s instructions detailing maintenance requirements.

      - Parts catalog that includes complete list of repair and replacement parts, with cuts and identifying numbers.

  - QUALITY ASSURANCE

    - Qualifications:

      - Worker experienced in performing work of this section who has specialized in work similar to that required of this project.

    - Regulatory Requirements.

Specifier Note: Manually operated projection screens must meet requirements of building codes and zoning bylaws issued by federal, state and local government authorities having jurisdiction (AHJs). Ensure that project specification section reflects the need to meet these requirements. Edit Article below as applicable.

      - Comply with [Uniform Building Code (UBC)] [International Building Code (IBC)] [Building code for [State] [City] of [______].

    - Preinstallation Meetings: Conduct preinstallation meeting to verify project requirements and manufacturer’s instructions. Comply with [Section 01 31 19 - Project Meetings] [______].

  - DELIVERY, STORAGE & HANDLING

    - Storage and Protection:

      - Store manually operated projection screens protected from exposure to harmful weather and in dry, ventilated conditions at temperature less than [80 degrees F (27 degrees C)] [______].

    - Handling: Handle manually operated projection screen materials with care in order to prevent damage.

    - Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

    - Waste Management and Disposal.

Specifier Note: The disposal of packaging waste into landfill sites demonstrates an inefficient use of natural resources and consumes valuable landfill space. Specifying appropriate packaging and construction waste management and disposal procedures may contribute to points required for LEED® construction project certification.

      - Separate waste materials for [reuse] [and] [recycling] [______] in accordance with [Section 01 74 19 - Construction Waste Management and Disposal] [______].

      - Remove packaging materials from site and dispose of at appropriate recycling facilities.

      - Collect and separate for disposal [paper] [plastic] [polystyrene] [corrugated cardboard] [______] packaging material [in appropriate onsite bins] [______] for recycling.

  - PROJECT AMBIENT CONDITIONS

    - Project Location: Perform manually operated projection screen work when temperatures are greater than [40 degrees F 4 degrees C)] [______].

  - SEQUENCING

    - Sequence With Other Work: Comply with projection screen manufacturer’s written recommendations for sequencing construction operations.

Specifier Note: Coordinate Article below with Contract Conditions and with Section [01 78 36 - Warranties] [______].

  - WARRANTY

    - Project Warranty: Refer to Contract Conditions for project warranty provisions.

    - Manufacturer’s Warranty: Submit, for Owner’s acceptance, manufacturer’s standard warranty document executed by authorized company official. Manufacturer’s warranty is in addition to, and does not limit, other rights Owner may have under Contract Documents.

Specifier Note: Coordinate Article below with manufacturer’s warranty requirements.

    - Warranty: Commencing on date of acceptance by [Owner] [Architect] [Consultant] [______].

  - MAINTENANCE MATERIALS

    - Use only standard product line parts that are produced by manufacturer of manually operated projection screens.

- PRODUCTS

  - MANUFACTURERS

    - Ensure manufacturer has minimum [5] [______] years experience manufacturing components similar to or exceeding project requirements.

    - Manufacturer: Da-Lite Screen Company, Inc.

      - Contact: P.O. Box 137, 3100 N. Detroit St., Warsaw, IN 46581-0137; Telephone: (800) 622-3737, (574) 267-8101; Fax: (877) 325-4832, (574) 267-7804; E-mail: info@da-lite.com; website: www.da-lite.com.

Specifier Note: Retain Article below for proprietary method specification. Add product attributes, performance characteristics, material standards and descriptions as applicable. Use of such phrases as “or equal,” “or approved equal” or similar phrases may cause ambiguity in specifications. Such phrases require verification (procedural, legal and regulatory) and assignment of responsibility for determining “or equal” products.

  - PROPRIETARY PRODUCTS/PROJECTION SCREEN SYSTEMS

Specifier Note: Determining the projection screen best suited for a project is based on many variables, including room configuration, projector placement, projection method, ambient light conditions, display brightness, projected image makeup (number of pixels) and screen maintenance expectations. These conditions should also be considered when determining screen placement and the surface required for optimum viewing.

    - Screen Type 2: Heavy Duty Manual Screens with viewing areas to 160 inches (4064 mm) wide.

      - Screen Operation: Manually operated, retractable projection screen mounted on ball bearing rigid steel spring roller with controlled screen return (CSR) mechanism.

        - Form screen bottom into pocket holding tubular metal slat with attached steel pull bail.

        - Protect slat ends with heavy-duty plastic end caps.

        - Include [6 feet (1.8 m)] [______] pull cord.

      - Screen Mounting Type:  [Wall] [Ceiling] [Recessed ceiling] mounted with screen case.

      - Screen Case:

        - Material:

          - 21 gage steel.

          - Case is designed to receive mounting hardware and is sized to suit projection screen.

        - Design: Flat-backed case with heavy duty plastic end caps concealing roller ends with steel inner plates to support roller. End caps to form sturdy brackets for wall or ceiling installations.

        - Include built-in bumper stops to prevent slat wedging into case.

        - Length: [______].

        - Finish: Powder coated [white] [black] [______].

- Specifier Note: Retain the below paragraph if veneer was selected for screen case finish. Select a veneer option.

          - [Cherry] [Light Oak] [Medium Oak] [Mahogany] [Natural Walnut] [Heritage Walnut] [Honey Maple] [_____].

Specifier Note: Screen size varies by screen model. Refer to manufacturer’s technical data sheets to determine the appropriate screen size to suit project requirements.

      - Screen Size:

        - Viewing Area: H [______] inches × W [______] inches (H [______] × W [______] mm).

        - Overall Dimensions: H [______] inches × W [______] inches (H [______] × W [______] mm).

      - Acceptable Materials: Da-Lite Screen Company, Inc., Model C with CSR Projection Screen.

Specifier Note: Screen surfaces with black borders are standard; surfaces without borders are optional.

        - Screen Viewing Surface:

          - Permanently attached to roller.

          - Front projection, flame retardant, mildew resistant, fiberglass fabric, black backed [with] [without] standard black borders, easily cleaned with mild soap and water solution.

Specifier Note: Certain fabrics are seamless to 6 feet (1.8 m). Others are seamless to 10 feet (3.1 m).

          - Seams: Seamless when both screen dimensions are less than [6 feet (1.8 m)] [10 feet (3.1 m)].

Specifier Note: The higher the gain number, the brighter the image. Gain numbers generally vary from 1.0 - 2.4, depending on the surface materials of the screen.

          - Gain: To SMPTE RP 94-2000, [1.0] [1.1] [1.5] [2.4].

Specifier Note: The larger the viewing angle, the less need there is to be directly in front of the screen for proper viewing. Generally, viewing angles vary between 30 - 50 degrees and occur in 5-degree increments.

          - Viewing Angle: [30] [35] [50] [60].

Specifier Note: The screen format is expressed as a ratio of width/height. Specify the screen format that best suits the graphic display and project requirements.

          - Format: [NTSC or Video – 1.33:1] [HDTV – 1.78:1] [16:10 Wide – 1.60:1] [Square – 1:1] [______].

Specifier Note: There is no industry standard defining the different types of viewing surfaces for manual projection screens. Manufacturers have developed different viewing surfaces to differentiate between one projection screen model and the next. Check other manufacturers’ product literature for names of viewing surfaces. Da-Lite® Screen Company, Inc. manufactures a variety of viewing surfaces for manually operated projection screens meeting Type 2 Heavy Duty Manual requirements, including Matte White, High Contrast Matte White, Silver Matte, Video Spectra 1.5 and High Power®. Each of these viewing surfaces has a different combination of surface finish, surface construction, gain and viewing angle.

Specifier Note: Viewing surfaces manufactured by Da-Lite® Screen Company, Inc., for use with Screen Type 2 Heavy Duty Manual Screens have received the GREENGUARD Indoor Air Quality Certification® (certification #90068) indicating that these surfaces meet performance based standards to define goods with low chemical and particle emissions for use indoors. These viewing surfaces have also received the GREENGUARD Children and Schools Certification indicating that these surfaces comply with the State of California’s Department of Health Services Standard Practice (CA Section 01350) for testing chemical emissions from building products used in schools. As such, GREENGUARD Children and Schools Certified products can be used as a strategy to earn valuable credits in the CHPS Best Practices Manual for K-12 schools. Use of these viewing surfaces may also contribute points toward LEED® certification. Refer to manufacturer’s technical data sheets to determine the most appropriate viewing surface to meet project requirements and insert the viewing surface name in the following paragraph. Specify High Contrast Matte White with GREENGUARD Certification #90068-1, High Power® with GREENGUARD Certification #90068-2, Matte White with GREENGUARD Certification #90068-3, Silver Matte with GREENGUARD Certification #90068-4 and Video Spectra 1.5 with GREENGUARD Certification #90068-5.

          - Acceptable Materials: Da-Lite Screen Company, Inc., [______] Viewing Surface.

Specifier Note: Optional accessories available for Screen Type 2 Large Venue Manual Screens with viewing area to 160 inches (4064 mm) vary by model. Refer to manufacturer’s technical data sheets to identify accessories available with specified projection screen system model.

      - Accessories:

Specifier Note: Extra screen drop is available to lower picture area more than normal. Calculate screen drop accordingly to ensure that maximum total surface height, including picture area, does not exceed 13 feet (4 m).

        - Screen Drop: Drop of [______] at [top] [bottom] with [black] [white] fabric.

        - Pull Rod: 38 inches (965 mm), zinc plated with plastic handle grip.

        - Installation Hardware: Fasteners and other components of type, size and spacing recommended by manufacturer for complete, functional and secure installation of projection screen.

Specifier Note: Retain the following paragraph if mounting accessories are needed for Model C or Model C with CSR manual screens.

        - Wall Brackets:

          - Fixed Length: Extends [6 inches (152 mm)] [10 inches (254 mm)] [14 inches (356 mm)] [15 3/4 inches (400 mm] [18 1/4 inches (464 mm) [20 inches (508 mm)] [24 inches (610 mm)]

          - Color: [Black] [White].

          - Capacity: 75 lb (34 kg) per pair maximum.

        - Floating Mounting Brackets: [Black] [White], recommended for drywall applications.

        - Ceiling Trim Kit, recommended for recessed ceiling applications.

  - PRODUCT SUBSTITUTIONS

    - Substitutions: [In accordance with Section 01 25 13 - Product Substitution Procedures] [______]] [No substitutions permitted].

- EXECUTION

  - INSTALLERS

    - Provide experienced and qualified technicians to install manually operated projection screens.

  - MANUFACTURER'S INSTRUCTIONS

Specifier Note: Article below is an addition to the CSI SectionFormat and a supplement to this section. Revise Article below to suit project requirements and specifier’s practice.

    - Compliance: Comply with manufacturer’s written data, including product technical bulletins, product catalog installation instructions, product carton installation instructions and Da-Lite Screen Company, Inc., technical data sheets.

  - EXAMINATION

    - Site Verification of Conditions:

      - Verify that conditions of substrates previously installed under other sections or contracts are acceptable with manually operated projection screen installation.

      - Inform [Owner] [Architect] [Consultant] [______] of unacceptable conditions immediately upon discovery.

      - Proceed with installation only after unacceptable conditions have been remedied.

  - COORDINATION

    - Coordinate projection screen placement with other ceiling and wall mounted components.

  - INSTALLATION

    - Install projection screens in accordance with reviewed shop drawings at locations and heights indicated.

      - Verify locations with [Owner] [Architect] [Consultant] [______] prior to installation.

      - Suspend screen from [ceiling support framing] [roof structure] [secondary support framing].

    - Install screen housing [prior to] [in conjunction with] installation of [suspended] [______] ceiling system.

    - Securely install screens plumb and level to supporting substrate.

  - FIELD QUALITY CONTROL

Specifier Note: Use the following Articles to verify the quality of installed components when manufacturer’s field services are desired. Establish the number and duration of periodic site visits required by the Manufacturer and specify below. Consult with Manufacturer for services required. Delete if field services are not required.

    - Manufacturer’s Field Services: Have manufacturer’s technical representative schedule site visits to review work as follows:

      - After delivery and storage of products.

      - When preparatory work for which work of this Section depends is complete, but before installation begins.

      - [Weekly] [2 times] [______] during progress of work [at [25%] and [60%]] [______] of completion.

      - Upon completion of work, after cleaning is carried out.

    - Inspection and Testing: Operate each screen [3] [______] times to ensure viewing surfaces extend and retract through full range of motion.

      - Ensure controlled sequence return (CSR) operates quietly and that other components function as designed and meet project requirements.

  - FINAL CLEANING

    - Perform cleanup in accordance with Section [01 74 00 - Cleaning and Waste Management] [______].

    - Upon completion, remove surplus materials, rubbish, tools and equipment.

  - PROTECTION

Specifier Note: Coordinate the following Article with Section 01 76 00 - Protecting Installed Construction.

    - Protect manually operated projection screens from damage during construction in accordance with Section [01 76 00 - Protecting Installed Construction] [______].

    - Repair damage to adjacent materials caused by manually operated projection screen work.

  - MAINTENANCE

    - Perform work during regular trade working hours satisfactory to [Owner] [Architect] [Consultant] [______].

END OF SECTION